STANDARD OPERATING PROCEDURE - #UCC005

Title: Storage of Genetically Modified Micro-Organisms (GMMs) in a Class 1 or Class 2 Laboratory in UCC

Effective Date:

Approvals (Signature and Date):

Author (Signature)	Technical Authority (Signature) Date: 11/06/2010

1.0	PURPOSE

  - This SOP outlines the storage of Class 1 and Class 2 GMMs in UCC.

2.0	SCOPE

2.1	This SOP applies to all labs in UCC where GMM/GMO work takes place.

3.0	RESPONSIBILITY

3.1	It is the responsibility of those working with GMMs & GMOs to be aware of precautions when working with such material. Lab Stock Storage Folder (for Fridge/Freezer/Cold Room) is to be maintained by Lab Manager or designated responsible person. It is the responsibility of each individual to update the folder on a regular basis with the location and details of their GMM/GMO stocks.

4.0	MATERIALS AND EQUIPMENT

4.1		Permanent marker for identification of tubes/plates. Clearly defined storage area for GMMs with Biohazard sign on entrance to area. Folders with templates for recording fridge/freezer/cold room location, box positions and box contents.

- HEALTH AND SAFETY CONSIDERATIONS

5.1	The H&S considerations when dealing with GMM/GMOs are critical to researcher and environment alike. Please store your GMM material in a safe manner.

6.0	PROCEDURE

6.1	Tubes/Plates are to be clearly labelled with permanent marker.

6.2	Fridge/Freezer/Cold Room Stock Storage Folder should be updated with location of GMM material. Details such as Fridge/Freezer location, Shelf number, Rack number, Box number and Row/Column details where appropriate (Please see Table 3 as an example of GMM Storage Record).

6.3	-80oC Freezers are to be maintained in good working order; filters are to be cleaned every three months. Should the digital display fall below -75oC degrees, a service call is to be placed to access the nature of the deviation/defect. The freezer should be serviced annually and service records should be retained for future inspections.

6.4	The entrance to the GMM storage area should have a biohazard sign (Please see Figure 1 as an example of Biohazard sign).

Table 3 Example of GMM storage record

| # GMM Storage Record |  |  |  |
| --- | --- | --- | --- |
| Date: | Name: | GMM Details | ###### Location |

##### BIOHAZARD

***Authorised Personnel only***

| Biosafety Level | _____________________________ |
| --- | --- |
| Principal Investigator | _____________________________ |
| Contact Details | _____________________________ |
| Authorization for visitor access must be obtained from the Principal Investigator listed above |  |
| Do not remove items from this lab without prior authorization Please keep this door locked when not attended |  |

Figure 1 Biohazard symbol for entrance to Laboratory and/or Growth facility/Animal houses.